JOSHUA TREE NATIONAL PARK

ABANDONED MINES SURVEY

SUMMER / FALL 2010

HOLISTIC WILDLIFE LLC

RICK SHERWIN

REPORT PREPARED BY CARL JOHANSSON

MINE SITE DESCRIPTIONS:

SNOW CLOUD MINE COMPLEX  JOTR S -109 – OP1,   OP4

Snow Cloud consists of a 200 foot deep steep decline 0p1, with 200 feet of workings on the bottom level, with carbide smoke writings dated 1952. There are  75 feet of side workings ( 60 on one side of the shaft, and 15 feet on the other side) at the second level approximately 130 feet from the surface.  The upper level OP 4 is a 125 foot adit terminating at OP4,  that intersects OP1 at the 75 foot deep level.  There are some curious side working in the OP4 adit, with an 8 foot deep sump that has a winze on it,  and an ore shoot-  less than 3 feet long that dumped ore from  a stope above the cribbing located 70 feet from the opening onto the adit level.

Liberal amounts of guano were present throughout the mine on all 3 levels.  I judged it to be from at least 3 species, based on size, color and shape of the guano pellets.  I suspect that it receives regular use as a day roost, with perhaps limited use as a night roost.  I detected no evidence of hibernacula or maternity use.

CONTACT MINE COMPLEX   JOTR S-011 -  OP1,  OP2,  OP3

Contact consisted of the main shaft OP 3, that appears to be where the vast majority of the mining took place. There are rails coming out of that shaft at an angle.  It appears as if those rails were used to skid ore to the surface.  OP1, and OP 2 as well as a few other opening are open stopes along a shallow adit that probably originally came off the main shaft, but was later worked by stoping to the surface.  The ground around these openings is unstable, and apparently there has been significant erosional closings as well as possible sloughing naturally filling in the openings and opening others along the adit.   Entry down the main shaft is precluded by safety concerns with a haystacking of wood timbers rails and other materials occurring about 40 feet down the shaft.

Because of the entry issues we have not internally surveyed this mine.

NEW EL DORADO MINE COMPLEX JTOR 074 – OP 1, 2, 4, 5

New El Dorado was a complex where processing of ore took place as well as mining.  There are remnants of water tanks, cement settling ponds, buildings and building platforms scattered throughout the site.  There is some confusion in my mind as to the numbering of openings, and if they have numbered each opening. We have reGPSed all of the openings we found.

OP 1 & 2 are open “pits” funnel shaped with lots of erosional fill.  They are connected by a 10 foot long drift.  Both also have drifts trending to the south, but both have been closed by erosional inflow.

OP 5 was identified by Kristen, a park biologist, as an opening not on the inventory.  It was a 25 foot deep shaft that drifted at the bottom toward OP 1 & 2.  However that drift has been naturally closed by erosional inflow.

What we identified as OP 3, a 30 foot long  adit at the bottom of the arroyo (identified as OP 4 on prioritized list) is an active day roost.  Scorpion pinchers were found indicating Antrozous pallidus use.  We saw a bat fly out of this adit when we started to survey it, and plastic laid on this visit (Sept 25th)  revealed significant guano collected 2 weeks later (Oct 25th).

STANDARD MINE COMPLEX  JTOR S-62 OP 3, 4, 5, 6, 7

Standard is a large complex mine site straddling the Joshua Tree National Park boundries.  The mine gets significant visitation, and appears to still be prospected as we observed 20 bags of cement and other signs of mining or preparing to mine the site.  The points we were given did not match up to our GPS so we reGPSed the positions and took our best guess as to what opening number corresponded to which opening.  OP 7 most likely was the shaft that went down about 50 feet with an intermediate level that was reconnected to the bottom level by a winze.  It was unsafe to venture to the bottom level because of haystacking .  OP 3, 4, & 5 were all connected as a main level adit, with OP4 being a stope to the surface.  Op 6 was an upper level adit that joined at an upper level, the stoped out incline accessed through OP 5.

Light to moderate amounts of guano was scattered throughout all of the openings and mines.

LUCKY TURKEY MINE COMPLEX  JTOR- 053 – OP 1, 3

Lucky Turkey is a complex consisting of a 225 foot adit  (OP3), that meets up with a 45 foot deep vertical shaft(OP1) at the end of the adit. There is an Arastra setup just down the hill from the shaft opening.  There are the remains of at least 2 “miners towns” within a quarter mile of the mine.  There is seasonal water in the mine, flowing down the shaft during periods of runoff.

There are several guano rings and substantial piles, which I suspect are maternity sites.  Each ring is located directly under any where the mine had any stoping.  There are also substantial amounts of insect parts scattered throughout the mine.  Plastic sheeting was placed to quantify seasonal use.

OLD SWEDE MINE COMPLEX JOTR –S-070- OP 1, 2, 3

Old Swede is a complex of 3 discrete shafts in the same area. The mines have previouskly been identified as roost sites for California Leaf-nosed Bats, which we observed flying in the area, Construction of gates for each of the mines was ongoing and subsequently off gassing and other construction issues made it difficult to establish normal occupancies.    Plastic was laid in each of the aditsto allow for quantifying of post gate installation and continues seasonal use.